Contents

1. Command structure	1

1.1 Structure of Sending command from server	1

1.2 Device response data structure	2

2. V1 GPS packet：	2

3. V3 data（LBS data）	3

4. Login data	4

5. HeartBeat	4

6. GPRS Command explaination	5

7. General Data Definition：	5

Data Packet Type

| HTBT | Heartbeat packet |
| --- | --- |
| V1 | GPS Packet |
| V3 | LBS Packet |
| V4 | Confirm packet |

# 1. Command structure

## 1.1 Structure of Sending command from server

*XX,YYYYYYYYYYYYYYY,CMD,HHMMSS,PARA1,PARA2,…#

*				command head

XX				supplier name, ASCII character

,				separator

YYYYYYYYYYYYYYY	15digit IMEI number

CMD			              command code

HHMMSS		              time：h/m/s

PARA			              parameter

#				ending character

Must use capital letter for the initial Letter of command character without any space.

## 1.2 Device response data structure

General information：

*XX,YYYYYYYYYYYYYYY,V1,HHMMSS,S,latitude,D,longitude,G,speed,direction,DDMMYY,equ _status#

*HQ,865205030330012,V1,145452,A,2240.55181,N,11358.32389,E,0.00,0,100815,FFFFFBFF#

GPS data：

*XX,YYYYYYYYYYYYYYY,V2,HHMMSS,S,latitude,D,longitude,G,speed,direction,DDMMYY,equ_status#

*HQ,865205030330012,V2,150421,A,2240.55841,N,11358.33462,E,2.06,0,100815,FFFFFBFF#

Confirm data：

*XX,YYYYYYYYYYYYYYY,V4,CMD,hhmmss,HHMMSS,S,latitude,D,longitude,G,speed,direction,DDMMYY,equ_status #

*HQ,865205030330012,V4,S2,150950,151007,A,2240.55503,N,11358.35174,E,0.85,0,100815, FFFFFBFF #

# 2. GPS packet - V1

*				command head

XX				Supplier name,ASCII character

,				separator

YYYYYYYYYYYYYYY	15digit IMEI number

V1/V2/V3/V4                                comamnd code

CMD			              to be confirmed command

hhmmss			              time value of confirmed command

HHMMSS		              Time

S			             data valid byte （A/V/B），

A represent valid data signal, V represent invalid data signal, B represent Beidou valid data signal

Latitude			              latitude，format DDMM.MMMMM

D				latitude symbol（N：north，S：south）

Longitude		              Longitude，format DDDMM.MMMMM

G				Longitude symbol（E：East，W：west）

Speed	 range 000.00 ~ 999.99  byte  （must be 3 digits XXX, if speed is 60km/h, so it need reserve a “space” in front of 60 value）

Direction			direction,true north is 0 degree,resolution 1 dgree, clockwise

DDMMYY		              date/month/year

equ_status		            （refer < General data definition >）

#                                                      ending

PS:prococol calculate method is decimal system. Calculate by knot. It require transformation when analyze speed in your platform. So can not calculate by km/h directly.   For example: if it show 10 in your platform, the real speed is 10 knot/h, 10*1.852=18.52km/h.

# 3. LBS packet - V3

*XX,YYYYYYYYYYYYYYY,V3,HHMMSS,Base_Info,Battery_Info,Failure_Info,Cont,DDMMYY,equ_status#

*				command head

XX				Supplier name,ASCII character

,				separator

YYYYYYYYYYYYYYY	15digit IMEI number

CMD			              comamnd code

HHMMSS		              terminal time

Base_Info：Base station information format asbelow:

MCC,MNC,Base_Number,LAC1,Cell_ID1,RS1,dBm1, LAC2,Cell_ID2,RS2,dBm2,…

MCC                                              country code, china is 460

MNC                                              network code, china mobile 00, China Union 01

Base_Number                                Cell ID quantity, 00-99

LAC                                               Base station area code

Cell_ID                                          Cell ID number

RS                                                  signal strength, the data show empty if no signal

dBm                                                receiving signal strength, the data show empty if no signal

Battery_Info                                  battery info, 0x0000-0x0299 hexadecimal represent battery voltage

Failure_Info                                  reboot info, 0-9

Cont                                               ending of data for extend protocol “X” represent DDMMYY

equ_status		            （refer<General data definition>）

#                                                     End

*HQ,865205030330012,V3,000201,46000,07,009350,004022,132,-88,009350,004032,140,,009350,004031,139,,009350,004023,133,,009350,004033,127,,009350,004021,124,,010351,003942,118,,0256,0,X,010915,FFFFFBFF#

Explain：

1. the data represent receiving 7 Cell ID signal, battery voltage 0x256/0x400=598/1024=0.584*5.6V =3.27V,  battery level is 3.27V/3.6V*100%=90.8% without reboot.

2. Battery percentage nominal voltage is 3.6V, battery percentage top and button limitation  1%-100%， if the upload voltage over than battery percentage top and button limitation, it regards to parse the data based on battery percentage top and button limitation.

V5 Data packet (WIFI data , For special GPS model support WIFI Tracking Only）

*XX,YYYYYYYYYYYYYYY,V5,HHMMSS,Wifi_Number, Wifi_Info, Battery_Info,DDMMYY,equ_status#

*				command head

XX				Supplier name,ASCII character

,				separator

YYYYYYYYYYYYYYY	IMEI number

CMD			              command code

HHMMSS		              device time

Wif_Number                                  wifi quantity，0-8

Wifi_Info：	front 12 digits represent MAC address, the later 2 digits represent signal strength of WIFI

Battery_Info                                  battery info, 0x0000-0x0299 hexadecimal represent battery voltage

equ_status		            （refer<General data definition >）

DDMMYY                                    day/month/year

#                                                     end

*HQ, 135790246811220,V5 ,8,11223344556677,11223344556677,11223344556677, 11223344556677,11223344556677,11223344556677,11223344556677,11223344556677, 0256,010815, FFFFFBFF#

# 4. HeartBeat - HTBT

*HQ, YYYYYYYYYYYYYYY,HTBT,Battery percent,remain uploading times#

For example：

Device upload：*HQ,135790246811220,HTBT,100,1200#

Server response：

*HQ,135790246811220,HTBT,100#

If show 100 means battery level 100%, 90 means battery level 90%

1200 means totally 1200 pings can be sent, 1199 means totally 1199 pings can be uploaded.

# 5. ICCID of SIM card

Device upload:    *HQ,865770062606670,ICCID,89860121801609042193#

Device response: *HQ.865770062606670,ICCID#

# 6. GPRSCommand explaination -CMD

Except special declare, server need response to sender.

Format: command

Command : refer to SMS command list

Response：

*XX, YYYYYYYYYYYYYYY,V4,CMD,,command,reply#

Command : command content,reply is the replied content.

The following is only examples. For more GPRS commands, please directly send SMS commands. The GPRS sending format is exactly the same as the SMS format.

Example：

Command： check123456

Reply：

*HQ,135790246811220,V4,CMD,check,CHECK,IMEI:861358034573915,APN:internet,IP:www.gps228.com:8989,C:,GSM:31,GPS:1,ACC:1,TZ:+8:30,ACCON/ACCOFF:5/60,SP:80,1,LMT2:40,Pw:On,12062,KC:1,OIL:90%,BAT:80%#

Example：

Command： reset123456

Reply：

*HQ,135790246811220,V4,CMD,reset,RESET OK#

Example for working mode：

Command：MD123456 1 3 8 1  （default mode 1）

Command：MD123456 2 3 1:00 2:00 （mode 2）

Command：MD123456 3 3 120   （mode 3）

# 7. General Data Definition：

equ_statusVehicle status: total 4 bytes, means vehicle status and alarm status etc. Use ASCII character represents Hexadecimal values, the following is the exact meaning of single bit of varite each byte, bit represents adopt negative logic, namely bit=0 is valid.

Important: Need convert to Binary format to get the Bit value; Check the converted data as arrow from end to front

as shown below table：

| Bit order | 1st byte |  | 2nd byte |  | 3rd byte |  | 4th byte |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | reserved | 1 | reserved | 1 | reserved | 1 | reserved |
| 1 | 1 | reserved | 1 | reserved | 1 | reserved | 1 | reserved |
| 2 | 0 | Removal alarm | 1 | reserved | 1 | reserved | 0 | Overspeed alarm |
| 3 | 1 | reserved | 1 | reserved | 0 | Vibration alarm | 1 |  |
| 4 | 1 | reserved | 1 | reserved | 0 | Low battery alarm | 0 | Fence-in alarm |
| 5 | 1 | reserved | 1 | reserved | 1 | reserved | 1 | reserved |
| 6 | 1 | reserved | 1 | reserved | 1 | reserved | 1 | reserved |
| 7 | 1 | reserved | 1 | reserved | 1 | reserved | 0 | Fence-out alarm |

Removal alarm example

How to parse status packet “FBFF FFFF”?