OBD GPS TRACKER

Specification

| Specification | parameters |
| --- | --- |
| SIZE | 47.4*33.2*27.6mm |
| WEIGHT | 32.5g (include battery) |
| INTERNAL BATTERY | 100mAH（3.7V） |
| Working voltage | 12-36V DC |
| GSM+GPS model | MT2503 |
| Working temperature | -20°C to+60°C |
| Restore temperature | -20°C to+70°C |
| sensor | Precision vibration sensor |
| Frequency band | Quad-band：850/900/1800/1900MHZ |
| GPRS | Class2,TCP/IP build in GSM Module |
| Root mean square error | S degree |
| Max output power | GSM 850/900:33.0±2dbm GSM 1800/1900:30.0±2dbm |
| Dynamic output range | -15~-108dbm |
| Receiving sensitivity | -107dbm |
| Maximum frequency error | ±0.1ppm |
| sensitivity | Code start：-145 dbm Hot start：-156 dbm Normal temperature start：-145 dbm navigation：-160 dbm track：-162 dbm |
| Location accuracy | ＜5m |
| First time location | Cold start：normal 32 S |
| GSM antenna | internal |
| GPSantenna | internal |
| LED light | Power source, status |
| Transfer protocol | TCP |
| Power detection | Report when main power disconnected |
| Data report | Report real time data |
| E-fence | TBD |
| Over speed alarm | Over speed alarm, (need setting) |
| GPS data | Update in real time |
| Vibration alarm | Report vibration alarm when setup defence. |