Protocol

The device ready , try to connect server will send the data to register

Message Body Length      that's means: message body data length

0100  terminal register

7E 01 00 00 27 01 38 33 50 42 79 00 13 00 2C 01 2F 37 30 31 31 31 42 53 4A 2D 41 36 2D 42 44 00 00 00 00 00 00 00 00 00 00 00 33 35 30 34 32 37 39 01 4E 58 63 7E

7E      header

01 00    main signaling

00 27    Message Body Length

01 38 33 50 42 79  Tracker SN number （the same with you set on the platform.    11bytes）

00 13    Serial Code(number)

00 2C     provincial ID

01 2F     city ID

37 30 31 31 31    Manufacturer ID

42 53 4A 2D 41 36 2D 42 44 00 00 00 00 00 00 00 00 00 00 00    terminal type

33 35 30 34 32 37 39    terminal ID(The factory default)

01   plate color

4E 58     plate

63  checksum

7E  Ending

0x8100： Platform Registration Reply：

7E8100000D013150284894000000010001020304050607080910087E

7E    header

8100  main signaling

000D   Message Body Length

013150284894   Tracker SN number （the same with u set on the platform.11bytes）

0000    Serial Code(number)

0001    Response serial number

00      result (0=success   1=vehicle already registered

01020304050607080910     Authentication number

08    checksum

7E     Ending

0102    terminal  authentication

7E0102000A0138335042790014010203040506070809106D7E

7E     header

0102   main signaling

000A    Message Body Length

013833504279     Tracker SN number （the same with u set on the platform.11bytes）

0014     Serial Code(number)

01020304050607080910     Authentication number

6D    checksum

7E     Ending

0x8001：  Platform Common Answer

7E80010005013150284894 00010002010200107E

7E       header

8001     main signaling

0005     Message Body Length

013150284894      Tracker SN number （the same with u set on the platform.11bytes）

0001       Serial Code(number)

0002       Response serial number

0102       ResponseID

00      result

10      checksum

7E      Ending

0200  location data

7E 02 00 00 2A 01 38 33 50 17 44 00 19 00 00 00 00 00 00 00 C2 01 59 7F A8 06 CC 01 58 00 80 00 00 00 08 15 08 18 07 21 21 01 04 00 00 02 F5 02 02 00 00 03 02 00 00 9F 7E

7E                  Header
02 00               main signaling
00 2A               Message Body Length
01 38 33 50 17 44     Tracker SN number （the same with u set on the platform.11bytes）
00 19               Serial Code(number)
00 00 00 00          Alarm Bit
00 00 00 C2          Status Bit

01 59 7F A8          Latitude

01 59 7F A8  hex to decimal  22642600

Decimal 22642600 ÷ 1000000 = 22.6426, if want to converted it to format of degree and minute, please : 22degree, 0.6426*60=38.556 minute----- so the Latitude is 2238.5560N

06 CC 01 58         Longitude

06 CC 01 58 hex to decimal 114032984

Decimal 114032984 ÷ 1000000 = 114.032984  If want to converted it to format of degree and minute, please: 114 degree,  0.032984*60=1.97904 minute----- so the longitude is 11401.9790N

00 80              Altitude 128m  
00 AA              Speed 170/10=17km/h   Default GPS speed, for reference
00 08              direction  8
15 08 18 07 21 21    time 2015-08-18 07:21:21

Extension bit data as bellow:   01 04 00 00 02 F5 02 02 00 00 03 02 00 00 9F

01[Mileage]         04[4 bytes mileage]         00 00 02 F5 means 757/10=75.7km
02[fule lever]        02[2bytes fuel]             00 00    fuel level no data
03[sensor speed]     02[2bytes sensor speed]     00 00    sensor speed no data  （gps or pulse speed, at the same time if both effective, take pulse rate, or, take gps speed
9F                 XOR checksum 
7E                 Ending

0x8001：  Platform Common Answer

7E80010005013150284894 00010002020000107E

7E       header

8001     main signaling

0005     Message Body Length

013150284894      Tracker SN number （the same with u set on the platform.11bytes）

0001       Serial Code(number)

0002       Response serial number

0200       ResponseID

00      result

10      checksum

7E      Ending

0002 heardbeat data

7E000200000138335042790016757E

7E     header

0002   main signaling

0000   Message Body Length

013833504279     Tracker SN number （the same with you set on the platform.11bytes）

0016     Serial Code(number)

75     checksum

7E    Ending

0x8001：  Platform Commonl Answer

7E80010005013150284894 00010002000200107E

0704  bilnd area data format

7E0704014301356777777707F5000801002600000000000000030159741206CBFD5001720000000116052709062401040000001D03020000002600000000000000030159746606CBFD50016B0000000116052709065301040000001D0302000000260000000000000003015975E406CBFE58013F0000000116052709072701040000001D0302000000260000000000000003015976DE06CBFF10012E0000000116052709075601040000001D0302000000260000000000000003015976BC06CBFED0012D0000000116052709083001040000001D0302000000260000000000000003015976FE06CBFEC001080000000116052709090001040000001D0302000000260000000000000003015976FE06CBFE7800FC0000000116052709093301040000001D0302000000260000000000000003015977A606CBFF3001080000000116052709100201040000001D030200001F7E

7E   header

0704 main signaling

0143 command loop code----message data length

013567777777 Trakcer SN number

07F5  Serial Code(number)

0008  The number of data items

01    data type. fixed. no need deal

0026  one location message data length

00000000000000030159741206CBFD5001720000000116052709062401040000001D03020000

0026 one location message data length

00000000000000030159746606CBFD50016B0000000116052709065301040000001D03020000

0026 one location message data length

0000000000000003015975E406CBFE58013F0000000116052709072701040000001D03020000

0026 one location message data length

0000000000000003015976DE06CBFF10012E0000000116052709075601040000001D03020000

0026 one location message data length

0000000000000003015976BC06CBFED0012D0000000116052709083001040000001D03020000

0026 one location message data length

0000000000000003015976FE06CBFEC001080000000116052709090001040000001D03020000

0026 one location message data length

0000000000000003015976FE06CBFE7800FC0000000116052709093301040000001D03020000

0026 one location message data length

0000000000000003015977A606CBFF3001080000000116052709100201040000001D03020000

1F  XOR checksum

7E  ending

Response：7E8001000501356777777707F507F5070400A37E

response:

7E header

8001 main signaling

0005 command loop code---message data length

013567777777 Tracker SN number

07F5 serical code(number)

07F5 reponse serial number

0704 response ID

00  result

A3  XOR checksum

7E  ending

0107 Extended protocol

7E0107001E0380000498370049143839353230323030313831323038343230313066035831310456312E301C7E

7E   header

0107  main signaling

001E  message body length

038000049837  Tracker SN number （the same with IMEI    11bytes）

0049  serial code(number)

14    ICCID length 20

3839353230323030313831323038343230313066        ICCID

03  mode length

583131  model X11

04    version length

56312E30  version V1.0

1C7E      checksum